# 🏎️ Atomic RC MRX Master Edition — Build Profile

# COPILOT INSTRUCTIONS

This document is structured for RC car testing, analysis, and reference. Please use the following guidelines whenever you answer questions or analyze data in this file:

## DOCUMENT STRUCTURE:

- This document uses multiple Heading 1 sections, each serving a specific purpose:

  - Run Log: Contains all test sessions and their results.

  - Inventory Overview: Lists all available parts, options, and configurations (e.g., springs, tires, chassis, electronics) that may be referenced or changed during runs.

  - Setup Tips: Contains tuning advice, troubleshooting steps, and best practices for car setup.

  - Tracks: Details the tracks/environments where the car may be tested or raced, including surface, layout, and any special considerations.

  - Rules: Lists event or class rules that must be followed for legal race setups.

  - Additional Heading 1 sections may be added as needed for other reference information.

- Each Session Title (e.g., 2025-12-31) is a Heading 2 under Run Log.

- Each session includes:

  - Run Baseline Setup (details before the run)

  - Run Summary (table of setup changes, lap data, feedback)

  - Each row in the Run Summary table represents a new test segment, starting with a setup change.

## HOW TO USE THIS DOCUMENT:

- When I ask questions, always reference the latest session (the last Heading 2 under Run Log) unless I specify otherwise.

- Use the Run Summary table to analyze consistency, lap times, and feedback.

- Reference the Inventory Overview for available parts and options when making setup recommendations.

- Use Setup Tips for tuning advice and troubleshooting.

- Reference the Tracks section to tailor suggestions for the current or specified track.

- Check the Rules section to ensure all recommendations are legal for the intended event or class.

- Use my heading styles (Heading 1, Heading 2, etc.) to navigate and extract relevant data from all sections.

- Always ground your answers in the structure and content of this document.

## RUN SUMMARY TABLE ANALYSIS (AI PROMPT LOGIC):

When analyzing or summarizing a session’s Run Summary table, Reference the Data under the Run Data section for this session.  Use the following definitions and calculations:

The data under the Run Data section is structured as follows: Lap#) time   [optional trailing number]

The trailing number indicates a “consistent lap.” If no number is present, it is not consistent.

Using this data, produce a Final Summary table with the following metrics:

Repeat = Number of consistent laps “out of” total laps

Repeat Percentage = Repeat expressed as a percentage out of 100%

Fast Lap = The single fastest lap time in the dataset

Avg = Average of all laps that are 7.000 seconds or faster (ignore any lap > 7.000)

Avg Qty = Count of all laps that are 7.000 seconds or faster

Present the results in a clean motorsports-style table with the five metrics listed above. Do not include any other commentary unless I ask for it.

## When asked for a summary or analysis:

- Present a table or list with Repeat, Repeat Percentage, Fast Lap, Avg, and Avg Qty for each segment.

- Highlight patterns in consistency, lap times, or handling.

- Compare new results to previous runs to identify what worked and what didn’t.

- Suggest next setup changes based on the latest run data and available options.

- Ensure all recommendations comply with the Rules section if preparing for a race or event.

- Adapt advice based on track details and setup tips as documented.

## EXPECTATIONS:

- Analyze trends, improvements, or issues based on my data and feedback.

- Suggest next setup changes based on the latest run data and available options.

- Highlight patterns in consistency, lap times, or handling.

- If possible, compare new results to previous runs to identify what worked and what didn’t.

- Ensure all recommendations comply with the Rules section if preparing for a race or event.

- Adapt advice based on track details and setup tips as documented.

Note: This section acts as a living prompt for Copilot. Update it as needed to reflect any changes in how you want your data organized, referenced, or analyzed.

End of Copilot Instructions section.

# 📦 Inventory Overview

## 🏎️ Chassis

- Atomic RC MRX Master Edition (base chassis)

- Optional Aluminum Chassis — MRXME-UP10 (12 g)

- Optional Brass Chassis — MRXME-04 (weight missing)

- MRX Master Side Wing (Small – 58 mm width) — weight missing

## 🛠️ Suspension & Handling

### Center & Rear

- Center Damper w/ Droop Adjustment — MRXME-UP08

- Center Damper Mount for 24 mm Battery — MRXME-UP09

- MRX Master Aluminum Knuckles V2

### Vertical Side Springs (optional)

- Soft — Green — MRXME-UP01S

- Medium — Red — MRXME-UP01M

- Hard — Yellow — MRXME-UP01H

### DAA Front Springs (optional)

- Super Soft — Red — MRXME-UP02SS

- Soft — White — MRXME-UP02S

- Medium — Yellow — MRXME-UP02M

- Hard — Purple — MRXME-UP02H

### Rear Top Spring (optional)

- Soft — White — MRXME-UP05S

- Medium — Yellow — MRXME-UP05M

- Hard — Purple — MRXME-UP05H

### Damper Greases (optional)

- Atomic Shock Grease: 15000 / 25000 / 35000 / 45000

- Reflex Racing Damper Grease: 250000

- Kyosho Differential Grease: 1000 / 3000 / 5000 / 15000 / 30000

## 🛞 Tires & Wheels

### Front Tire Options:

- Kyosho 30 (Medium/Hard) (Yellow)

- Marka V5 Front 25° / 11 mm (Medium) (White)

- Gekko FM-GK-002 (Medium) (Red)

- Gekko FS-GK-001 (soft) (Black)

### Rear Tire Options:

- Kyosho 30 (Medium/Hard) (Yellow)

- Kyosho 20 (Medium) (Red)

- Gekko RM1-GK-004 (Medium) (Gray)

- Marka MZR-V1RR15 (Medium/soft) (White)

- Gekko RS1 GK-003 (Soft) (Black)

### Front Wheel Options:

- Silver Horse EVO “JUD” — 8.5 mm +1 offset

- Silver Horse EVO “JUD” — 8.5 mm +0 offset

- Silver Horse EVO “JUD” — 8.5 mm -1 offset

### Rear Wheels

- Silver Horse EVO “JUD” — 11 mm +3 offset

- Silver Horse EVO “JUD” — 11 mm +2 offset

- Silver Horse EVO “JUD” — 11 mm +1 offset

- Silver Horse EVO “JUD” — 11 mm +0 offset

### ⚡ Electronics & Power

- Motor: PN Anima V4 2500kv (non-handout, for practice)

- ESC: Hobbywing EZRun Mini28

- Servo: AGFRC A06CLS v2

- Battery: Silver Horse RC 450 mAh 2S 7.4V 60C (3.33 Wh)

- Radio: Flysky Noble 4 Plus

- Receiver: Micro 4-channel

- Lap Timing: EasyLap Transponder

# 🛠️ Setup Notes (Baseline)

- Built per MRX Master Edition manual

### Front suspension:

  - Front Ride Height: 2.1

  - Front Damper Grease: Atomic 25000

  - Front DAA Springs: Super Soft (red)

  - Toe Angle: 4 degrees out

  - Top Kingpin Shim: 0

  - Bottom Kingpin Shim: 2

  - Caster: 2

  - Damper Shim: 1

  - Axles: v2

### Rear Suspension:

  - Rear Ride Height: 3.2

  - Style: Horizontal

  - Center Damper Grease: : Atomic 25000

  - Side Spring Damper Grease: Atomic 25000

  - Vertical Side Springs: n/a

  - Rear Top Spring: Medium (hard)

### Drive

  - Pinion: 12T

  - Spur Gear: Atomic Stock 53T

  - Diff: Atomic Gear Diff (Stock)

  - Motor: PN 2500kv Anima v4

  - Tires:

    - Front: Marka V5 Front 25° / 11 mm

    - Rear: Marka MZR-V1RR15-14

  - Wheels:

    - Front: Silver Horse EVO “JUD” — 8.5 mm +1 offset

    - Rear: Silver Horse EVO “JUD” — 11 mm +3 offset

# Setup Tips:

### 🔧⚙️Ball Differential Tuning Advice

- Tighten the ball diff fully, then back off about 1/8 to 1/4 turn to allow smooth operation without slipping.

- The diff should turn smoothly by hand without grit or binding.

- Avoid overtightening, which can cause the diff to act like a locked diff and reduce smooth differential action.

- If the diff slips or makes a barking sound under power, tighten it slightly.

- Regularly check and lubricate the diff with silicone or diff grease for optimal performance and longevity.

- Adjust tightness based on track conditions and personal driving feel.

# Tracks

### 🏟️ Track in the Cave — Test Environment

- Location: Basement garage

- Dimensions: 12 ft × 24 ft

- Surface: RCP 30 cm tiles (smooth side up)

- Layout:

  - Two 12 ft straights

  - One 24 ft straight

  - Road-course style

- Purpose: FX28 + MRX tuning, consistency testing, setup validation

This is a perfect environment for F1 chassis testing — long enough to expose stability issues, narrow enough to punish sloppy steering, and consistent enough to track setup changes.

# Rules

## PNWC

### 🏁 PNWC Formula 1 Class — Compliance Summary

Your MRX Master Edition is legal for PNWC F1.

Key PNWC Requirements (and your status)

| Rule Area | PNWC Requirement | Your Build |
| --- | --- | --- |
| Chassis | MRZ F1 allowed | ✔ MRX Master Edition (MRZ family) |
| Body | Must use approved F1 bodies | ✔ You can run PN, GL, G-Force, ZP06, Grivi, or kit body |
| Wings | Must use original kit wings | ✔ You have original MRX wings |
| Motor | Handout PN V4 Anima 2500kv | ⚠ Practice motor OK, race requires handout |
| Gearing | Fixed 12/53 (64 pitch) | ✔ You can run this |
| ESC | Any, no gyro | ✔ Mini28 is legal |
| Battery | 2S LiPo, max 8.40V | ✔ Your pack is legal |
| Wheels | PN MR2088/2089 Delrin ONLY | ⚠ Your Silver Horse wheels are NOT legal for PNWC |
| Tires | Mini-Z rubber only | ✔ Marka rubber is legal |
| Weight | ≥ 140 g | ⚠ Need to confirm final weight |

The only PNWC-critical change you’ll need later is switching to PN Delrin wheels.

# 📘 Run Log

## Test 2025-12-28

### Run Baseline Setup:

| ##### Front suspension: - Front Ride Height: 2.1 - Front Damper Grease: Atomic 25000 - Front DAA Springs: Yellow (Med) - Toe Angle: 4 degrees out - Caster: Top Kingpin Shim: 0 - Caster: Bottom Kingpin Shim: 2 - Camber: 2 - Damper Shim: 1 - Axles: v2 | ##### Rear Suspension: - Rear Ride Height: 3.2 - Style: Horizontal - Center Damper Grease: : Atomic 45000 - Side Spring Damper Grease: Atomic 25000 - Vertical Side Springs: n/a - Rear Top Spring: Medium (Yellow) + 0 Turn | ##### Drive - Pinion: 12T - Spur Gear: Kyosho 53T - Diff: Kyosho Ball Diff - Motor: PN 2500kv Anima v4 ##### Tires & Wheels - Front: Gekko FM-GK-002 (Med) (Red) ¦ +1offset - Rear: Gekko RM1-GK-004 (Med) (Gray) ¦ + 3 offset |
| --- | --- | --- |

### Run Summary:

| Setup Change | Start | End | Repeat | Fast | Avg | Feedback |
| --- | --- | --- | --- | --- | --- | --- |
| New Battery / New Tires | 1 | 22 | 11 / 22 | 6.2 | 6.5(13) | Steering wide and unpredictable |
| Front DAA Springs: White (Soft) | 23 | 62 | 7 / 30 | 6.2 | 6.7 (11) | Steering wide and unpredictable |
| Front DAA Springs: Red (Super Soft) | 63 | 74 | 8 /12 (66%) | 6.2 | 6.8 (9) | Steering wide and unpredictable |
| Front Tires: Gekko FS-GK-001 (soft) (Black) | 75 | 78 | 1 / 4 (25%) | n/a | n/a | Steering better, but bites hard – car flips |
| Front Ride Height: 1.8 | 79 | 94 | 4 / 16 (25%) | 6.5 | 6.6 (5) | Bites hard – car flips |
| Gekko FM-GK-002 (Med) (Red) | 95 | 108 | 2 / 14 (14%) | 6.6 | 6.8 (2) | Bites hard – car flips in turns |
| Marka V5 Front 25° / 11 mm (Medium) (White) Marka MZR V1RR15 (Medium/soft) (White) Front Ride Height not measured | 109 | 146 | 17 / 38 (44%) | 5.8 | 6.4 (23) | Could be driver – Steering |
| Front Ride Height: 1.8 | 147 | 174 | 18 / 28 (64%) | 5.8 | 6.2 (24) | Drives great – Looking for what if |
| Front DAA Springs: White (Soft) Front Ride Height: 1.8 | 175 | 207 | 18 / 33 (54%) | 6.0 | 6.2 (18) | Steering not as predictable |
| Front Ride Height: 2.0 | 208 | 229 | 10 /22 (45%) | 6.1 | 6.3 (13) | Steering more predicable but car is slower |
| Front Ride Height: 2.1 (by damper) | 230 | 249 | 12 / 20 (60%) | 5.9 | 6.2 (16) | Battery died |
| New Battery | 250 | 281 | 16/32 (50%) | 5.9 | 5.9 (22) | Drives Great – Looking for what if |
| Front DAA Springs: Yellow (Med) Front Ride Height: 2.1 | 282 | 301 | 10/20 (50%) | 5.8 | 6.0 (13) | Drives Great – Looking for more speed |
| Toe Angle: 2 degrees out | 302 | 362 | 21/61 (34%) | 5.8 | 6.2 (38) |  |
| New Battery | 363 | 395 | 14/33 (44%) | 5.8 | 6.3 (15) | Steering is improved but unpredictable on throttle |
| Rear Diff – Tight – ¼ turn | 396 | 401 | 1 / 6 (16%) | 6.3 | 6.3 (3) | Diff Too loose |
| Rear Diff – Tight – 1/8 turn | 402 | 435 | 18/34 (52%) | 5.7 | 6.1 (20) | Drives Great – Looking for more speed |
| Center Damper Grease: : Atomic 15000 | 436 | 461 | 15/26 (57%) | 5.7 | 6.2 (17) | Drives Great – Looking for more speed |
| Rear Top Spring: Medium (hard) - 3 Turns | 462 | 505 | 24/44 (54%) | 5.8 | 6.2 (31) | Drives Great – Looking for more speed |
| Diff – Atomic gear diff (Stock) New Battery | 506 | 537 | 7/32 (21%) | 6.0 | 6.3 (16) |  |
| Rear Top Spring: Medium (Yellow) + 3 Turns | 538 | 587 | 20/50 (40%) | 5.9 | 6.2 (27) |  |
| Rear Top Spring: Medium (Yellow) + 0 Turns | 588 | 615 | 14/28 (50%) | 6.0 | 6.2 (20) |  |
| Marka V5 Front 25° / 11 mm (Medium) (White) (Swapped old tires with slightly used tires) | 616 | 682 | 35/67 (52%) | 6.0 | 6.4 (45) |  |

### Run Data

#### Lap 1 - 22

- 1) 0:07.489

- 2) 0:08.674

- 3) 0:09.044  1

- 4) 0:07.128

- 5) 0:06.562  1

- 6) 0:09.063

- 7) 0:09.655  1

- 8) 0:15.555

- 9) 0:08.561

- 10) 0:07.049

- 11) 0:10.146

- 12) 0:07.193

- 13) 0:07.462  1

- 14) 0:06.691

- 15) 0:06.582  1

- 16) 0:06.674  2

- 17) 0:06.440  3

- 18) 0:06.358  4

- 19) 0:06.638  5

- 20) 0:06.202  6

- 21) 0:06.952

- 22) 0:06.746  1

#### Lap 23 - 62

- 23) 27:41.523

- 24) 1:15.914

- 25) 0:08.800

- 26) 0:06.864

- 27) 0:10.316

- 28) 0:08.968

- 29) 0:07.027

- 30) 0:06.992  1

- 31) 0:06.861  2

- 32) 0:06.582  3

- 33) 0:23.352

- 34) 0:09.519

- 35) 0:06.374

- 36) 0:06.270  1

- 37) 0:09.286

- 38) 0:08.929  1

- 39) 0:06.427

- 40) 0:07.091

- 41) 0:06.640  1

- 42) 0:06.522  2

- 43) 0:07.396

- 44) 0:06.606

- 45) 0:20.112

- 46) 0:12.372

- 47) 0:07.040

- 48) 0:08.631

- 49) 0:06.667

- 50) 0:07.525

- 51) 0:10.844

- 52) 0:09.171

- 53) 0:07.158

- 54) 0:06.964  1

- 55) 0:06.637  2

- 56) 0:06.810  3

- 57) 4:59.728

- 58) 0:07.123

- 59) 0:07.010  1

- 60) 0:10.550

- 61) 0:11.409  1

- 62) 0:06.754

#### Lap 63 - 74

- 63) 0:06.948  1

- 64) 0:06.493  2

- 65) 0:06.862  3

- 66) 0:06.490  4

- 67) 0:06.853  5

- 68) 0:09.496

- 69) 0:06.800

- 70) 0:06.740  1

- 71) 0:06.254  2

- 72) 0:07.653

- 73) 0:08.312  1

- 74) 0:06.541

#### Lap 75 - 78

- 75) 8:55.048

- 76) 0:09.515

- 77) 0:09.110  1

- 78) 0:11.062

#### Lap 79 - 94

- 79) 0:08.597

- 80) 0:06.606

- 81) 0:06.883  1

- 82) 6:49.497

- 83) 0:07.786

- 84) 0:07.105  1

- 85) 0:14.756

- 86) 0:11.546

- 87) 0:06.739

- 88) 0:07.372  1

- 89) 0:06.629

- 90) 0:12.227

- 91) 0:10.357

- 92) 0:07.022

- 93) 0:06.544  1

- 94) 0:08.765

#### Lap 95 - 108

- 95) 0:16.142

- 96) 106:41.545

- 97) 0:09.968

- 98) 0:06.646

- 99) 0:14.276

- 100) 0:10.180

- 101) 0:08.187

- 102) 0:07.405  1

- 103) 0:18.982

- 104) 0:13.899

- 105) 0:09.841

- 106) 0:06.997

- 107) 0:08.551

- 108) 0:08.594  1

#### Lap 109 - 146

- 109) 30:32.928

- 110) 0:06.877

- 111) 0:07.043  1

- 112) 0:12.223

- 113) 0:08.186

- 114) 0:06.629

- 115) 0:06.768  1

- 116) 0:06.682  2

- 117) 0:07.867

- 118) 0:06.160

- 119) 0:11.298

- 120) 0:06.735

- 121) 0:06.500  1

- 122) 0:06.089  2

- 123) 0:06.149  3

- 124) 0:06.436  4

- 125) 0:06.870  5

- 126) 0:06.946  6

- 127) 0:06.717  7

- 128) 0:07.470

- 129) 0:11.428

- 130) 0:06.109

- 131) 0:05.852  1

- 132) 0:07.461

- 133) 0:08.633

- 134) 0:07.748

- 135) 0:07.503  1

- 136) 0:06.458

- 137) 0:06.067  1

- 138) 0:08.563

- 139) 0:06.195

- 140) 0:06.269  1

- 141) 0:08.415

- 142) 0:08.490  1

- 143) 0:05.951

- 144) 0:06.056  1

- 145) 0:05.951  2

- 146) 0:06.623

- 147) 12:21.901

#### Lap 147 - 174

- 148) 0:06.299

- 149) 0:07.563

- 150) 0:06.362

- 151) 0:06.022  1

- 152) 0:05.924  2

- 153) 0:06.318  3

- 154) 0:06.074  4

- 155) 0:06.339  5

- 156) 0:05.906  6

- 157) 0:06.905

- 158) 0:06.358  1

- 159) 0:06.116  2

- 160) 0:05.988  3

- 161) 0:05.805  4

- 162) 0:06.064  5

- 163) 0:06.980

- 164) 0:06.139

- 165) 0:08.267

- 166) 0:08.479  1

- 167) 0:06.044

- 168) 0:06.601  1

- 169) 0:05.901

- 170) 0:06.412  1

- 171) 0:06.052  2

- 172) 0:06.336  3

- 173) 0:06.082  4

- 174) 0:06.325  5

#### Lap 175 - 207

- 175) 8:02.898

- 176) 0:07.196

- 177) 0:07.925

- 178) 0:11.159

- 179) 0:07.119

- 180) 0:07.016  1

- 181) 0:07.277  2

- 182) 0:08.685

- 183) 0:06.772

- 184) 0:06.415  1

- 185) 0:06.309  2

- 186) 0:07.017

- 187) 0:07.441  1

- 188) 0:06.572

- 189) 0:07.574

- 190) 0:07.599  1

- 191) 0:06.093

- 192) 0:06.501  1

- 193) 0:06.446  2

- 194) 0:06.873  3

- 195) 0:06.368  4

- 196) 0:06.373  5

- 197) 0:06.138  6

- 198) 0:07.020

- 199) 0:06.067

- 200) 0:09.072

- 201) 0:06.218

- 202) 0:06.667  1

- 203) 0:06.295  2

- 204) 0:06.184  3

- 205) 0:06.274  4

- 206) 0:06.129  5

- 207) 0:06.032  6

#### Lap 208 - 229

- 208) 0:06.169  7

- 209) 0:06.192  8

- 210) 10:01.268

- 211) 0:08.455

- 212) 0:06.672

- 213) 0:08.776

- 214) 0:08.314  1

- 215) 0:06.388

- 216) 0:06.117  1

- 217) 0:07.009

- 218) 0:14.541

- 219) 0:06.169

- 220) 0:06.283  1

- 221) 0:06.273  2

- 222) 0:12.424

- 223) 0:09.359

- 224) 0:07.674

- 225) 0:06.483

- 226) 0:06.353  1

- 227) 0:06.151  2

- 228) 0:06.523  3

- 229) 0:06.221  4

#### Lap 230 - 249

- 230) 16:18.748

- 231) 0:06.225

- 232) 0:05.943  1

- 233) 0:06.156  2

- 234) 0:06.497  3

- 235) 0:06.241  4

- 236) 0:06.361  5

- 237) 0:06.042  6

- 238) 0:06.039  7

- 239) 0:06.191  8

- 240) 0:10.338

- 241) 0:06.536

- 242) 0:06.265  1

- 243) 0:06.164  2

- 244) 0:08.362

- 245) 0:06.126

- 246) 0:06.254  1

- 247) 0:09.562

- 248) 0:06.403

- 249) 0:06.429  1

#### Lap 250 - 281

- 250) 1:53.824

- 251) 0:07.713

- 252) 0:07.015  1

- 253) 0:08.343

- 254) 0:06.161

- 255) 0:08.068

- 256) 0:06.365

- 257) 0:08.887

- 258) 0:08.308  1

- 259) 0:06.715

- 260) 0:06.217  1

- 261) 0:09.015

- 262) 0:06.354

- 263) 0:07.004

- 264) 0:06.727  1

- 265) 0:05.954

- 266) 0:07.146

- 267) 0:10.104

- 268) 0:06.245

- 269) 0:06.497  1

- 270) 0:06.725  2

- 271) 0:06.872  3

- 272) 0:06.011

- 273) 0:05.963  1

- 274) 0:05.948  2

- 275) 0:06.057  3

- 276) 0:06.020  4

- 277) 0:06.134  5

- 278) 0:06.146  6

- 279) 0:06.458  7

- 280) 0:06.113  8

- 281) 0:06.095  9

#### Lap 282 - 301

- 282) 9:23.829

- 283) 0:09.121

- 284) 0:06.604

- 285) 0:08.539

- 286) 0:07.768  1

- 287) 0:06.249

- 288) 0:10.146

- 289) 0:08.127

- 290) 0:06.229

- 291) 0:06.218  1

- 292) 0:06.820  2

- 293) 0:08.561

- 294) 0:05.862

- 295) 0:06.216  1

- 296) 0:06.192  2

- 297) 0:05.997  3

- 298) 0:06.507  4

- 299) 0:06.100  5

- 300) 0:06.023  6

- 301) 0:06.022  7

#### Lap 302 - 362

- 302) 24:22.613

- 303) 0:06.638

- 304) 0:07.133  1

- 305) 0:07.998

- 306) 0:06.079

- 307) 0:06.311  1

- 308) 0:09.339

- 309) 0:06.154

- 310) 0:10.799

- 311) 0:06.133

- 312) 0:08.951

- 313) 0:09.816  1

- 314) 0:09.985  2

- 315) 0:05.932

- 316) 0:11.864

- 317) 0:06.024

- 318) 0:06.488  1

- 319) 0:07.850

- 320) 0:06.334

- 321) 0:05.816  1

- 322) 0:06.405

- 323) 0:07.389

- 324) 0:06.470

- 325) 0:07.549

- 326) 0:06.117

- 327) 0:05.947  1

- 328) 0:05.875  2

- 329) 0:07.233

- 330) 0:08.436

- 331) 0:21.937

- 332) 0:07.795

- 333) 0:06.007

- 334) 0:06.790

- 335) 0:10.485

- 336) 0:08.197

- 337) 0:12.608

- 338) 0:06.587

- 339) 0:06.422  1

- 340) 0:10.645

- 341) 0:06.378

- 342) 0:07.805

- 343) 0:06.486

- 344) 0:06.644  1

- 345) 0:06.992  2

- 346) 0:06.231

- 347) 0:06.403  1

- 348) 0:06.344  2

- 349) 0:05.959  3

- 350) 0:09.480

- 351) 0:06.273

- 352) 0:09.587

- 353) 0:06.621

- 354) 0:06.289  1

- 355) 0:06.084  2

- 356) 0:06.095  3

- 357) 0:07.551

- 358) 0:06.003

- 359) 0:06.051  1

- 360) 0:06.223  2

- 361) 0:06.787  3

- 362) 0:06.410  4

#### Lap 363 - 395

- 363) 2:12.198

- 364) 0:06.237

- 365) 0:06.915

- 366) 0:06.095

- 367) 0:06.021  1

- 368) 0:11.958

- 369) 0:33.268

- 370) 0:06.612

- 371) 0:10.793

- 372) 0:07.103

- 373) 0:07.790  1

- 374) 0:06.294

- 375) 0:11.874

- 376) 0:08.509

- 377) 0:08.300  1

- 378) 0:08.005  2

- 379) 0:07.991  3

- 380) 0:07.824  4

- 381) 0:06.203

- 382) 0:06.241  1

- 383) 0:06.100  2

- 384) 0:05.860  3

- 385) 0:06.375  4

- 386) 0:06.540  5

- 387) 0:06.085  6

- 388) 0:06.008  7

- 389) 0:07.462

- 390) 0:08.226

- 391) 0:14.151

- 392) 0:08.413

- 393) 0:06.005

- 394) 0:06.461  1

- 395) 0:09.385

#### Lap 396 - 401

- 396) 9:30.282

- 397) 0:06.404

- 398) 0:09.920

- 399) 0:06.459

- 400) 0:06.306  1

- 401) 0:10.256

#### Lap 402 - 435

- 402) 0:07.549

- 403) 0:07.514  1

- 404) 0:07.137  2

- 405) 2:12.787

- 406) 0:09.262

- 407) 0:08.300

- 408) 0:06.365

- 409) 0:08.073

- 410) 0:06.554

- 411) 0:07.657

- 412) 0:06.236

- 413) 0:06.776  1

- 414) 0:06.161  2

- 415) 0:06.415  3

- 416) 0:05.853  4

- 417) 0:07.156

- 418) 0:06.034

- 419) 0:06.039  1

- 420) 0:08.113

- 421) 0:08.741  1

- 422) 0:06.982

- 423) 0:06.213

- 424) 0:06.074  1

- 425) 0:05.887  2

- 426) 0:05.783  3

- 427) 0:06.036  4

- 428) 0:06.062  5

- 429) 0:06.074  6

- 430) 0:06.742

- 431) 0:07.042  1

- 432) 0:06.524  2

- 433) 0:06.837  3

- 434) 0:06.061

- 435) 0:05.995  1

#### Lap 436 - 461

- 436) 7:51.696

- 437) 0:09.013

- 438) 0:11.370

- 439) 0:09.199

- 440) 0:06.348

- 441) 0:06.146  1

- 442) 0:06.317  2

- 443) 0:05.754  3

- 444) 0:11.962

- 445) 0:10.822  1

- 446) 0:07.633

- 447) 0:06.349

- 448) 0:06.080  1

- 449) 0:06.367  2

- 450) 0:06.089  3

- 451) 0:06.085  4

- 452) 0:06.320  5

- 453) 0:06.108  6

- 454) 0:07.115

- 455) 0:06.502  1

- 456) 0:05.923  2

- 457) 0:07.094

- 458) 0:06.068

- 459) 0:06.039  1

- 460) 0:06.000  2

- 461) 0:06.210  3

#### Lap 462 - 505

- 462) 0:15.408

- 463) 0:09.278

- 464) 0:06.059

- 465) 0:06.102  1

- 466) 5:54.122

- 467) 0:13.294

- 468) 0:06.230

- 469) 0:06.374  1

- 470) 0:06.035  2

- 471) 0:05.998  3

- 472) 0:06.219  4

- 473) 0:06.145  5

- 474) 0:10.527

- 475) 0:06.540

- 476) 0:05.821

- 477) 0:06.120  1

- 478) 0:08.179

- 479) 0:06.248

- 480) 0:06.050  1

- 481) 0:06.079  2

- 482) 0:06.192  3

- 483) 0:06.757  4

- 484) 0:08.086

- 485) 0:08.638  1

- 486) 0:06.085

- 487) 0:08.005

- 488) 0:06.162

- 489) 0:06.511  1

- 490) 0:07.687

- 491) 0:06.341

- 492) 0:07.656

- 493) 0:06.079

- 494) 0:06.151  1

- 495) 0:06.666  2

- 496) 0:06.162  3

- 497) 0:05.864  4

- 498) 0:06.135  5

- 499) 0:06.709  6

- 500) 0:06.501  7

- 501) 0:06.202  8

- 502) 0:07.441

- 503) 0:06.386

- 504) 0:06.883  1

- 505) 0:06.366  2

#### Lap 506 - 537

- 506) 14:21.142

- 507) 0:07.657

- 508) 0:06.707

- 509) 0:06.214  1

- 510) 0:10.910

- 511) 0:08.896

- 512) 0:16.230

- 513) 0:06.528

- 514) 0:06.696  1

- 515) 0:09.632

- 516) 0:10.261  1

- 517) 0:07.255

- 518) 0:56.791

- 519) 0:10.624

- 520) 0:06.366

- 521) 0:21.068

- 522) 0:06.961

- 523) 0:06.242

- 524) 0:10.434

- 525) 0:06.168

- 526) 0:07.911

- 527) 0:06.478

- 528) 0:11.143

- 529) 0:06.384

- 530) 0:07.816

- 531) 0:06.491

- 532) 0:06.484  1

- 533) 0:10.519

- 534) 0:06.092

- 535) 0:06.086  1

- 536) 0:06.159  2

- 537) 0:06.058  3

#### Lap 538 - 586

- 538) 3:48.733

- 539) 0:07.350

- 540) 0:06.097

- 541) 0:06.343  1

- 542) 0:06.218  2

- 543) 0:06.518  3

- 544) 0:08.251

- 545) 0:06.039

- 546) 0:10.488

- 547) 0:06.056

- 548) 0:06.275  1

- 549) 0:06.406  2

- 550) 0:06.254  3

- 551) 0:08.933

- 552) 0:09.847

- 553) 0:07.477

- 554) 0:06.943  1

- 555) 0:45.876

- 556) 0:11.284

- 557) 0:08.278

- 558) 0:06.298

- 559) 0:06.311  1

- 560) 0:06.292  2

- 561) 0:06.333  3

- 562) 0:06.491  4

- 563) 0:05.962  5

- 564) 0:10.549

- 565) 0:06.622

- 566) 0:07.084  1

- 567) 0:06.342

- 568) 0:06.181  1

- 569) 0:06.499  2

- 570) 0:06.349  3

- 571) 0:06.448  4

- 572) 0:07.875

- 573) 0:07.096  1

- 574) 0:10.622

- 575) 0:14.280

- 576) 0:07.095

- 577) 0:09.674

- 578) 0:09.858  1

- 579) 0:08.155

- 580) 0:09.617

- 581) 0:07.779

- 582) 0:06.609

- 583) 0:09.101

- 584) 0:11.585

- 585) 0:08.421

- 586) 0:06.763

#### lap 587 - 615

- 587) 0:06.121  1

- 588) 0:06.467  2

- 589) 0:06.423  3

- 590) 0:06.585  4

- 591) 4:23.549

- 592) 0:29.423

- 593) 0:07.463

- 594) 0:09.313

- 595) 0:06.301

- 596) 0:12.177

- 597) 0:06.264

- 598) 0:06.370  1

- 599) 0:06.979  2

- 600) 0:06.832  3

- 601) 0:06.077

- 602) 0:06.233  1

- 603) 0:08.233

- 604) 0:09.167

- 605) 0:06.885

- 606) 0:06.385  1

- 607) 0:06.209  2

- 608) 0:06.476  3

- 609) 0:06.667  4

- 610) 0:10.275

- 611) 0:08.184

- 612) 0:06.587

- 613) 0:06.352  1

- 614) 0:06.307  2

- 615) 0:06.274  3

#### Lap 616 - 682

- 616) 16:58.708

- 617) 0:07.461

- 618) 0:07.774  1

- 619) 0:10.701

- 620) 0:06.374

- 621) 0:11.411

- 622) 0:06.723

- 623) 0:07.960

- 624) 0:06.684

- 625) 0:07.645

- 626) 0:07.203  1

- 627) 0:07.072  2

- 628) 0:13.786

- 629) 0:06.694

- 630) 0:06.428  1

- 631) 0:06.478  2

- 632) 0:06.520  3

- 633) 0:06.419  4

- 634) 0:06.490  5

- 635) 0:06.846  6

- 636) 0:06.534  7

- 637) 0:08.108

- 638) 0:09.757

- 639) 0:06.129

- 640) 0:07.368

- 641) 0:06.124

- 642) 0:06.347  1

- 643) 0:06.605  2

- 644) 0:07.784

- 645) 0:06.081

- 646) 0:06.359  1

- 647) 0:06.783  2

- 648) 0:06.187  3

- 649) 0:06.813

- 650) 0:06.747  1

- 651) 0:06.350  2

- 652) 0:06.398  3

- 653) 0:06.420  4

- 654) 0:10.587

- 655) 0:07.475

- 656) 0:06.139

- 657) 0:06.301  1

- 658) 0:06.147  2

- 659) 0:07.388

- 660) 0:07.120  1

- 661) 0:11.823

- 662) 0:08.775

- 663) 0:06.691

- 664) 0:07.890

- 665) 0:06.261

- 666) 0:06.389  1

- 667) 0:06.386  2

- 668) 0:06.906  3

- 669) 0:10.745

- 670) 0:06.998

- 671) 0:06.514  1

- 672) 0:09.640

- 673) 0:06.246

- 674) 0:06.362  1

- 675) 0:06.414  2

- 676) 0:06.373  3

- 677) 0:06.281  4

- 678) 0:06.871  5

- 679) 0:06.620  6

- 680) 0:06.130  7

- 681) 0:06.259  8

- 682) 0:06.436  9

## Test 2025-12-31

### Run Baseline Setup:

| ##### Chassis - Side wings: Installed - Chassis Weights: LR/RF w/ body LF/RR; w/ body LR/RF. | ##### Front suspension: - Front Ride Height: 2.2 - Front Damper Grease: Atomic 25000 - Front DAA Springs: Yellow (Med) - Toe Angle: 2 degrees out - Caster: Top Kingpin Shim: 0 - Caster: Bottom Kingpin Shim: 2 - Camber: 2 - Damper Shim: 1 - Axles: v2 | ##### Rear Suspension: - Rear Ride Height: 3.2 - Style: Horizontal - Center Damper Grease: Atomic 45000 - Side Spring Damper Grease: Atomic 25000 - Vertical Side Springs: Medium (Red) - Rear Top Spring: Medium (Yellow) + 0 Turn ##### Drive - Pinion: 12T - Spur Gear: Atomic 53T - Diff: Atomic Gear Diff - Diff Rebuilt: Kyosho 3k (green) & Rebuilt - Motor: PN 2500kv Anima v4 ##### Tires & Wheels - Front: Marka V5 Front 25° / 11 mm (Medium) (White) - Rear: Marka MZR V1RR15 (Medium/soft) (White) |
| --- | --- | --- |

### Run Summary:

| Setup Change | Start | End | Repeat | Fast | Avg | Feedback |
| --- | --- | --- | --- | --- | --- | --- |
| Run Baseline Setup w/New Battery Weights LR/RF | 1 | 103 | 46/103 (44%) | 6.136 | 6.536 (63) | Car drives very smooth and fun to drive. Want to find more speed. |
| Chassis weights LF/RR (New Battery) | 104 | 193 | 35/90 (40%) | 6.055 | 6.422 (57) | Car drives very smooth (Tires: LF: 21.8, RF: 21.2 (no discs on Front) LR: 24.8, RR: 24.7 |
| Front Ride height: 2.0 (Adjusted by Droop screw) (New Battery) | 194 | 254 | 20/61 (32%) | 6.034 | 6.430 (33) | Not as smooth, steering keeps darting left or right. |
| Front Ride height: 2.0 IAdjusted by Spring collar) (New Battery) | 255 | 309 | 22/52 (42%) | 6.015 | 6.305 (36) | Smoothness returned, car feels quicker. |
| Rear Top Spring: Hard (Purple) | 310 | 357 | 21/48 (44%) | 6.192 | 6.63 (21) | Rear of the car vibrates. The car sounds faster, but the lap times do not show it. |
| Center damper grease: Atomic 15000 | 358 | 410 | 27/53 (51%) | 6.005 | 6.43 (27) | Rear of the car vibrates. The car sounds faster, but the lap times do not show it. Forces me to drive hard instead of smooth. |
| Front Springs: Soft (white) Front Damper Grease: 25000 Front Ride Height: 2.0 | 411 | 431 | 10/21 (48%) | 6.194 | 6.4 (10) | Smooth in turns, car is not aggressive Had to stop due to dead battery. |
| New battery | 432 | 479 | 15/48 (31%) | 6.216 | 6.49 (15) | Steering servo getting stuck Left. Nothing stands out to cause this issue. Noticed RF tire showing plastic (21.0). LF measures 21.6 |
| Changed RF tire (21.5) | 480 | 504 | 8/25 (32%) | 6.255 | 6.56 (8) | Steering servo getting stuck Left. Nothing stands out to cause this issue. |
| Replaced servo Camber: 2 Toe Angle: 2 degrees out Front Ride Height: 2.0 New Battery | 505 | 567 | 11/63 (17%) | 6.150 | 6.60 (11) | Rear very loose. Temp is 64 and very cold. |
| Replace rear tires with MR04 Evo | 568 | 617 | 32/50 (64%) | 5.948 | 6.234 (32) | Car feels great. Believe a new set of rear tires would set new records. |

### Run Data

#### Lap 1 - 103

- 1) 0:12.132

- 2) 0:06.525

- 3) 0:06.967  1

- 4) 0:06.341  2

- 5) 0:06.670  3

- 6) 0:10.368

- 7) 0:08.090

- 8) 0:06.570

- 9) 0:08.821

- 10) 0:06.289

- 11) 0:06.597  1

- 12) 0:07.142  2

- 13) 0:06.795  3

- 14) 0:06.506  4

- 15) 0:06.757  5

- 16) 0:08.868

- 17) 0:06.664

- 18) 0:07.113  1

- 19) 0:08.052

- 20) 0:07.933  1

- 21) 0:08.771

- 22) 0:07.385

- 23) 0:07.613  1

- 24) 0:06.535

- 25) 0:06.750  1

- 26) 0:06.687  2

- 27) 0:06.605  3

- 28) 0:06.485  4

- 29) 0:06.701  5

- 30) 0:07.063  6

- 31) 0:06.281

- 32) 0:06.697  1

- 33) 0:06.519  2

- 34) 0:07.533

- 35) 0:11.997

- 36) 0:06.892

- 37) 0:09.452

- 38) 0:12.752

- 39) 0:10.091

- 40) 0:07.989

- 41) 0:06.646

- 42) 0:06.882  1

- 43) 0:06.698  2

- 44) 0:06.448  3

- 45) 0:07.104

- 46) 0:06.542  1

- 47) 0:06.833  2

- 48) 0:06.476  3

- 49) 0:06.733  4

- 50) 0:07.169  5

- 51) 0:06.473  6

- 52) 2:58.456

- 53) 0:09.247

- 54) 0:07.997

- 55) 0:11.701

- 56) 0:06.548

- 57) 0:08.200

- 58) 0:07.937  1

- 59) 0:06.670

- 60) 0:06.948  1

- 61) 0:07.488  2

- 62) 0:06.774  3

- 63) 0:08.712

- 64) 0:07.413

- 65) 0:06.799  1

- 66) 0:07.923

- 67) 0:07.760  1

- 68) 0:06.331

- 69) 0:07.522

- 70) 0:06.369

- 71) 0:06.186  1

- 72) 0:06.759  2

- 73) 0:06.252  3

- 74) 0:06.277  4

- 75) 0:06.619  5

- 76) 0:07.902

- 77) 0:07.063

- 78) 0:06.214

- 79) 0:06.461  1

- 80) 0:07.167

- 81) 0:06.517  1

- 82) 0:06.235  2

- 83) 0:06.686  3

- 84) 0:07.828

- 85) 0:06.357

- 86) 0:06.136  1

- 87) 0:06.692  2

- 88) 0:06.444  3

- 89) 0:06.388  4

- 90) 0:13.710

- 91) 0:09.200

- 92) 0:08.975  1

- 93) 0:06.703

- 94) 0:07.148  1

- 95) 0:07.040  2

- 96) 0:18.390

- 97) 0:07.850

- 98) 0:06.851

- 99) 0:11.887

- 100) 0:06.714

- 101) 0:07.477

- 102) 0:07.113  1

- 103) 0:28.522

#### Lap 104 - 193

- 104) 923:33.837

- 105) 0:09.689

- 106) 0:06.567

- 107) 0:07.178  1

- 108) 0:06.998  2

- 109) 0:07.569  3

- 110) 0:09.226

- 111) 0:06.300

- 112) 0:07.476

- 113) 0:06.404

- 114) 0:06.326  1

- 115) 0:08.352

- 116) 0:07.131

- 117) 0:07.997

- 118) 0:07.072

- 119) 0:06.294

- 120) 0:06.832  1

- 121) 0:17.885

- 122) 0:10.187

- 123) 0:08.502

- 124) 0:07.278

- 125) 0:06.818  1

- 126) 0:08.531

- 127) 0:06.620

- 128) 0:08.647

- 129) 0:06.549

- 130) 0:06.266  1

- 131) 0:06.175  2

- 132) 0:10.572

- 133) 0:06.132

- 134) 0:09.540

- 135) 0:06.743

- 136) 0:06.185  1

- 137) 0:06.260  2

- 138) 0:06.055  3

- 139) 0:07.462

- 140) 0:12.068

- 141) 0:09.710

- 142) 0:06.468

- 143) 0:07.157

- 144) 0:06.291

- 145) 0:06.174  1

- 146) 0:06.537  2

- 147) 0:06.084  3

- 148) 0:07.141

- 149) 0:09.576

- 150) 0:06.761

- 151) 0:06.177  1

- 152) 0:06.327  2

- 153) 0:06.166  3

- 154) 0:06.561  4

- 155) 0:10.330

- 156) 0:10.203  1

- 157) 0:08.300

- 158) 0:06.749

- 159) 0:07.183  1

- 160) 0:07.555  2

- 161) 0:06.584

- 162) 0:06.248  1

- 163) 1:04.340

- 164) 0:06.302

- 165) 0:07.145

- 166) 0:06.428

- 167) 0:06.363  1

- 168) 0:06.424  2

- 169) 0:10.033

- 170) 0:09.804  1

- 171) 0:06.781

- 172) 0:07.128  1

- 173) 0:09.133

- 174) 0:06.176

- 175) 0:08.055

- 176) 0:06.914

- 177) 0:06.182

- 178) 0:07.320

- 179) 0:06.353

- 180) 0:06.642  1

- 181) 0:07.303  2

- 182) 0:07.996  3

- 183) 0:06.285

- 184) 0:08.610

- 185) 0:07.382

- 186) 0:06.461

- 187) 0:07.291

- 188) 0:07.323  1

- 189) 0:06.831  2

- 190) 0:06.343  3

- 191) 0:06.225  4

- 192) 0:06.664  5

- 193) 0:07.118  6

#### Lap 194 – 254

- 194) 100:37.566

- 195) 0:08.607

- 196) 0:11.966

- 197) 0:09.508

- 198) 0:06.828

- 199) 0:09.225

- 200) 0:07.641

- 201) 0:32.863

- 202) 0:09.094

- 203) 0:06.326

- 204) 0:07.180

- 205) 0:09.373

- 206) 0:08.669  1

- 207) 0:06.237

- 208) 0:06.418  1

- 209) 0:06.509  2

- 210) 0:06.568  3

- 211) 0:06.996  4

- 212) 0:11.133

- 213) 0:07.569

- 214) 0:06.374

- 215) 0:06.213  1

- 216) 0:08.588

- 217) 0:06.641

- 218) 0:06.667  1

- 219) 0:09.301

- 220) 0:06.308

- 221) 0:08.042

- 222) 0:09.809

- 223) 0:06.841

- 224) 0:06.644  1

- 225) 0:07.355

- 226) 0:08.746

- 227) 0:06.511

- 228) 0:09.199

- 229) 0:14.007

- 230) 0:08.244

- 231) 0:06.034

- 232) 0:08.191

- 233) 0:09.305

- 234) 0:11.102

- 235) 0:06.884

- 236) 0:06.442  1

- 237) 0:06.152  2

- 238) 0:08.830

- 239) 0:06.427

- 240) 0:06.319  1

- 241) 0:06.953

- 242) 0:06.969  1

- 243) 0:06.385  2

- 244) 0:08.507

- 245) 0:06.683

- 246) 0:06.352  1

- 247) 0:06.290  2

- 248) 0:07.705

- 249) 0:06.487

- 250) 0:06.619  1

- 251) 0:06.225  2

- 252) 0:06.732  3

- 253) 0:06.624  4

- 254) 0:06.175  5

#### Lap 255 – 309

- 255) 20:13.593

- 256) 0:07.518

- 257) 0:08.318

- 258) 0:07.900  1

- 259) 0:06.926

- 260) 0:06.871  1

- 261) 0:06.534  2

- 262) 0:06.015  3

- 263) 0:07.831

- 264) 0:06.684

- 265) 0:06.543  1

- 266) 0:06.232  2

- 267) 0:07.766

- 268) 0:06.597

- 269) 0:06.193  1

- 270) 0:06.371  2

- 271) 0:10.130

- 272) 0:08.646

- 273) 0:06.618

- 274) 0:08.485

- 275) 0:08.911  1

- 276) 0:06.467

- 277) 0:07.405

- 278) 0:20.938

- 279) 0:06.036

- 280) 0:06.222  1

- 281) 0:06.630  2

- 282) 0:06.137  3

- 283) 0:06.078  4

- 284) 0:06.280  5

- 285) 0:06.039  6

- 286) 0:08.840

- 287) 0:06.395

- 288) 0:06.284  1

- 289) 0:07.751

- 290) 0:06.980  1

- 291) 0:06.154

- 292) 0:06.962

- 293) 0:06.125

- 294) 0:06.161  1

- 295) 0:06.248  2

- 296) 0:06.721  3

- 297) 0:07.339  4

- 298) 0:07.338  5

- 299) 0:06.282

- 300) 0:06.832  1

- 301) 0:06.264  2

- 302) 0:06.921

- 303) 0:06.144

- 304) 0:06.620  1

- 305) 0:06.207  2

- 306) 0:06.242  3

- 307) 0:06.398  4

- 308) 0:06.153  5

- 309) 0:06.128  6

#### Lap 310 – 357

- 310) 25:04.684

- 311) 0:07.276

- 312) 0:07.285  1

- 313) 0:06.412

- 314) 0:12.812

- 315) 0:09.203

- 316) 0:08.516  1

- 317) 0:06.414

- 318) 0:06.879  1

- 319) 0:07.021  2

- 320) 0:08.122

- 321) 0:09.920

- 322) 0:06.280

- 323) 0:35.251

- 324) 0:06.427

- 325) 0:34.605

- 326) 0:09.832

- 327) 0:06.441

- 328) 0:06.198  1

- 329) 0:06.551  2

- 330) 0:07.309

- 331) 0:07.161  1

- 332) 0:07.366  2

- 333) 0:06.966  3

- 334) 0:07.231  4

- 335) 0:06.711  5

- 336) 0:07.926

- 337) 0:06.315

- 338) 0:06.775  1

- 339) 0:09.102

- 340) 0:08.403  1

- 341) 0:06.306

- 342) 0:06.247  1

- 343) 0:06.320  2

- 344) 0:08.521

- 345) 0:06.587

- 346) 0:10.029

- 347) 0:06.192

- 348) 0:06.690  1

- 349) 0:06.350  2

- 350) 0:06.683  3

- 351) 0:08.640

- 352) 0:06.622

- 353) 0:09.357

- 354) 0:06.417

- 355) 0:06.453  1

- 356) 0:06.528  2

- 357) 0:07.047  3

#### Lap 358 - 410

- 358) 10:01.211

- 359) 0:07.270

- 360) 0:06.890  1

- 361) 0:06.772  2

- 362) 0:06.737  3

- 363) 0:07.628

- 364) 0:06.482

- 365) 0:06.476  1

- 366) 0:06.684  2

- 367) 0:06.258  3

- 368) 0:06.200  4

- 369) 0:09.542

- 370) 0:12.307

- 371) 0:06.084

- 372) 0:07.773

- 373) 0:07.448  1

- 374) 0:06.642

- 375) 0:06.504  1

- 376) 0:06.675  2

- 377) 0:06.456  3

- 378) 0:06.479  4

- 379) 0:11.492

- 380) 0:06.427

- 381) 0:06.463  1

- 382) 0:07.812

- 383) 0:08.074  1

- 384) 0:08.322  2

- 385) 0:06.781

- 386) 0:06.521  1

- 387) 0:06.402  2

- 388) 0:09.755

- 389) 0:09.530  1

- 390) 0:08.774  2

- 391) 0:06.577

- 392) 0:11.987

- 393) 0:06.610

- 394) 0:06.504  1

- 395) 0:06.450  2

- 396) 0:06.435  3

- 397) 0:06.414  4

- 398) 0:08.288

- 399) 0:06.396

- 400) 0:08.708

- 401) 0:06.466

- 402) 0:06.364  1

- 403) 0:06.573  2

- 404) 0:06.781  3

- 405) 0:08.344

- 406) 0:07.107

- 407) 0:07.177  1

- 408) 0:06.382

- 409) 0:07.079

- 410) 0:07.096  1

#### Lap 411 – 431

- 411) 25:39.796

- 412) 0:07.030

- 413) 0:07.964

- 414) 0:07.660  1

- 415) 0:07.547  2

- 416) 0:07.641  3

- 417) 0:06.514

- 418) 0:06.593  1

- 419) 0:06.333  2

- 420) 0:07.048

- 421) 0:06.390  1

- 422) 0:06.317  2

- 423) 0:06.317  3

- 424) 0:06.662  4

- 425) 0:07.002  5

- 426) 0:08.071

- 427) 0:06.692

- 428) 0:06.194  1

- 429) 0:06.400  2

- 430) 0:06.264  3

- 431) 0:06.822  4

#### Lap 432 – 479

- 432) 6:30.662

- 433) 0:08.410

- 434) 0:07.145

- 435) 0:06.464  1

- 436) 0:06.858  2

- 437) 0:07.650

- 438) 0:07.020  1

- 439) 0:06.438  2

- 440) 0:07.713

- 441) 0:09.345

- 442) 0:06.447

- 443) 0:07.730

- 444) 0:08.566

- 445) 0:11.011

- 446) 0:09.635

- 447) 0:06.216

- 448) 0:11.544

- 449) 0:10.388

- 450) 0:07.312

- 451) 0:08.990

- 452) 0:06.567

- 453) 0:07.308

- 454) 0:09.901

- 455) 0:09.227  1

- 456) 0:06.468

- 457) 0:06.585  1

- 458) 0:10.731

- 459) 0:06.536

- 460) 0:15.748

- 461) 0:07.396

- 462) 0:06.422

- 463) 0:06.583  1

- 464) 0:08.804

- 465) 0:07.859

- 466) 0:06.527

- 467) 0:08.021

- 468) 0:06.405

- 469) 0:06.506  1

- 470) 0:07.264

- 471) 0:13.036

- 472) 0:07.934

- 473) 0:08.183  1

- 474) 0:06.437

- 475) 0:08.733

- 476) 0:06.583

- 477) 0:06.268  1

- 478) 0:06.498  2

- 479) 0:06.653  3

#### Lap 480 – 504

- 480) 11:03.114

- 481) 0:33.655

- 482) 0:10.219

- 483) 0:08.538

- 484) 0:11.465

- 485) 0:08.838

- 486) 0:07.568

- 487) 0:07.314  1

- 488) 0:06.567

- 489) 0:06.255  1

- 490) 0:06.669  2

- 491) 0:07.697

- 492) 0:09.921

- 493) 0:06.778

- 494) 0:07.180  1

- 495) 0:07.886  2

- 496) 0:06.267

- 497) 0:07.109

- 498) 0:08.950

- 499) 0:06.321

- 500) 0:06.844  1

- 501) 0:06.868  2

- 502) 0:15.271

- 503) 0:06.548

- 504) 0:11.989

#### Lap 505 – 567

- 505) 193:38.707

- 506) 0:19.777

- 507) 0:18.081  1

- 508) 185:30.585

- 509) 0:31.654

- 510) 0:10.646

- 511) 0:10.950  1

- 512) 0:12.153

- 513) 0:07.687

- 514) 0:07.743  1

- 515) 0:07.877  2

- 516) 0:08.563  3

- 517) 0:07.810  4

- 518) 0:09.782

- 519) 0:12.178

- 520) 0:07.684

- 521) 0:09.111

- 522) 0:08.615  1

- 523) 0:10.142

- 524) 0:06.616

- 525) 0:09.371

- 526) 0:06.501

- 527) 0:07.062  1

- 528) 0:12.126

- 529) 0:06.085

- 530) 0:06.525  1

- 531) 0:11.164

- 532) 0:07.884

- 533) 0:09.956

- 534) 0:08.645

- 535) 0:06.590

- 536) 0:06.498  1

- 537) 0:20.666

- 538) 0:10.466

- 539) 0:06.313

- 540) 0:07.524

- 541) 0:08.371

- 542) 0:10.201

- 543) 0:07.481

- 544) 0:06.354

- 545) 0:06.508  1

- 546) 0:10.730

- 547) 0:07.291

- 548) 0:07.112  1

- 549) 0:09.275

- 550) 0:08.546  1

- 551) 0:08.666  2

- 552) 0:06.150

- 553) 0:08.787

- 554) 0:06.458

- 555) 0:09.608

- 556) 0:08.082

- 557) 0:06.948

- 558) 0:09.079

- 559) 0:06.832

- 560) 0:12.391

- 561) 0:07.852

- 562) 0:08.611  1

- 563) 0:07.488

- 564) 0:07.169  1

- 565) 0:06.824  2

- 566) 0:07.140  3

- 567) 0:06.527  4

#### Lap 568 – 617

- 568) 10:26.589

- 569) 0:09.136

- 570) 0:06.791

- 571) 0:06.183  1

- 572) 0:06.137  2

- 573) 0:10.304

- 574) 0:08.942

- 575) 0:06.147

- 576) 0:06.160  1

- 577) 0:06.060  2

- 578) 0:06.847

- 579) 0:06.453  1

- 580) 0:06.250  2

- 581) 0:06.669  3

- 582) 0:06.187  4

- 583) 0:06.738  5

- 584) 0:06.089  6

- 585) 0:06.068  7

- 586) 0:05.948  8

- 587) 0:13.909

- 588) 0:06.029

- 589) 0:12.220

- 590) 0:08.964

- 591) 0:07.723

- 592) 0:06.629

- 593) 0:08.271

- 594) 0:06.139

- 595) 0:06.063  1

- 596) 0:06.015  2

- 597) 0:06.043  3

- 598) 0:11.384

- 599) 0:06.191

- 600) 0:06.059  1

- 601) 0:06.670

- 602) 0:06.324  1

- 603) 0:06.147  2

- 604) 0:06.213  3

- 605) 0:06.289  4

- 606) 0:06.258  5

- 607) 0:06.443  6

- 608) 0:06.157  7

- 609) 0:06.143  8

- 610) 0:06.260  9

- 611) 0:06.269  10

- 612) 0:06.027  11

- 613) 0:06.662

- 614) 0:07.224  1

- 615) 0:06.129

- 616) 0:06.148  1

- 617) 0:06.191  2

#### Lap 618 -

The only PNWC-critical change you’ll need later is switching to PN Delrin wheels.

## Test 2026-01-03

### Run Baseline Setup:

| ##### Chassis - Side wings: Installed - Chassis Weights: LR/RF w/ body LF/RR; w/ body LR/RF. | ##### Front suspension: - Front Ride Height: 2.0 - Front Damper Grease: Atomic 25000 - Front DAA Springs: Soft (white) - Toe Angle: 2 degrees out - Caster: Top Kingpin Shim: 0 - Caster: Bottom Kingpin Shim: 2 - Camber: 2 - Damper Shim: 1 - Axles: v2 | ##### Rear Suspension: - Rear Ride Height: 3.2 - Style: Horizontal - Center Damper Grease: Atomic 15000 - Side Spring Damper Grease: Atomic 25000 - Vertical Side Springs: Medium (Red) - Rear Top Spring: Hard (Purple) + 0 Turn ##### Drive - Pinion: 12T - Spur Gear: Atomic 53T - Diff: Atomic Gear Diff - Diff Rebuilt: Kyosho 3k (green) & Rebuilt - Motor: PN 2500kv Anima v4 ##### Tires & Wheels - Front: Marka V5 Front 25° / 11 mm (Medium) (White) - Rear: Marka MZR V1RR15 (Medium/soft) (White) |
| --- | --- | --- |

### Run Summary:

| Setup Change | Start | End | Repeat | Fast | Avg | Feedback |
| --- | --- | --- | --- | --- | --- | --- |
| Run Baseline Setup w/New Battery Weights LR/RF | 1 | 103 | 46/103 (44%) | 6.136 | 6.536 (63) | Car drives very smooth and fun to drive. Want to find more speed. |

### Run Data

#### Lap 1 - 103

- 1) 0:12.132

- 2) 0:06.525

- 3) 0:06.967  1

- 4) 0:06.341  2

- 5) 0:06.670  3

- 6) 0:10.368

- 7) 0:08.090

- 8) 0:06.570

# END OF DOCUMENT